11/15/2025

Milestone # 3

Robotics (Edpt1009) W’21 course project

Robotics (EDPT1009)

Project Milestone: MS#3

Team Number: #02

Chess Playing Robotic Arm

Marwan Ashraf Saad, Adam Mohamed Ismail, Ahmed Amr Abdelhay, Ayman Tarek, Bassel Mostafa Mahran, Youssef Nader Greis

# Inverse Position Kinematics

The MATLAB script implements an iterative numerical inverse kinematics (IK) algorithm to find the robot's joint angles that achieve a desired end-effector position.

The specific method is a form of the Newton-Raphson algorithm (also known as the Jacobian-based method). Here's how it works:

- Forward Kinematics (FK): The algorithm starts with an initial guess for the joint angles. It calculates the robot's current end-effector position by multiplying the transformation matrices T1 through T4.

- Position Error: It computes the error, which is the difference between the desired position and the current position.

- The Jacobian: The core of this method is the Jacobian matrix. This matrix describes how small changes in the joint angles will cause small changes in the end-effector's position. Your script calculates this numerically by slightly moving each joint one by one and seeing how the end-effector position changes.

- Inverting the Relationship: The algorithm's goal is to find the change in joint angles needed to correct the position error. It uses the Jacobian to solve for this required change.

- Pseudo-Inverse: Since the robot has 4 joints and is only trying to control 3 position coordinates (x, y, z), the Jacobian matrix isn't square. A standard inverse can't be used. The script uses a "pseudo-inverse" (the pinv function in MATLAB). This is a mathematical tool that finds the most efficient change in joint angles to correct the error.

- Iteration and Update: The joint angles are then updated by adding the calculated change (scaled by the alpha value). This entire process (Steps 1-6) repeats in a loop.

This loop continues until the position error is smaller than the specified tolerance, meaning the robot's end-effector has successfully reached the target position.

Iter   1 | Error = 0.452840 m

Iter   2 | Error = 0.422518 m

Iter   3 | Error = 0.392861 m

Iter   4 | Error = 0.364811 m

Iter   5 | Error = 0.339347 m

Iter   6 | Error = 0.317417 m

Iter   7 | Error = 0.299830 m

Iter   8 | Error = 0.287114 m

Iter   9 | Error = 0.279383 m

Iter  10 | Error = 0.275685 m

Iter  11 | Error = 0.270621 m

Iter  12 | Error = 0.265295 m

Iter  13 | Error = 0.258711 m

Iter  14 | Error = 0.251456 m

Iter  15 | Error = 0.244101 m

Iter  16 | Error = 0.237181 m

Iter  17 | Error = 0.227795 m

Iter  18 | Error = 0.215003 m

Iter  19 | Error = 0.200409 m

Iter  20 | Error = 0.184416 m

Iter  21 | Error = 0.167971 m

Iter  22 | Error = 0.150418 m

Iter  23 | Error = 0.131678 m

Iter  24 | Error = 0.111802 m

Iter  25 | Error = 0.090889 m

Iter  26 | Error = 0.069083 m

Iter  27 | Error = 0.046632 m

Iter  28 | Error = 0.024307 m

Iter  29 | Error = 0.009886 m

Iter  30 | Error = 0.003962 m

Iter  31 | Error = 0.001587 m

Iter  32 | Error = 0.000635 m

Iter  33 | Error = 0.000254 m

Iter  34 | Error = 0.000102 m

Iter  35 | Error = 0.000041 m

Converged after 35 iterations.

Final Joint Angles (deg):

q1 = 78.6881°

q2 = -44.1315°

q3 = 85.4983°

q4 = 99.7213°

Final End-Effector Position:

x = 0.0500, y = 0.2500, z = 0.2000

Final Error Norm = 0.000041 m

Final Jacobian Jv =

-0.2500   -0.0219    0.0022   -0.0188

0.0500   -0.1093    0.0108   -0.0939

0    0.2549   -0.1574    0.0551

# Forward and Inverse Velocity Kinematics

## Symbolic Jacobian J (per-degree)

[ (pi*sin((pi*q1)/180)*(l4*sin((pi*(q2 - q3 + q4))/180) + l2*sin((pi*q2)/180) + l3*sin((pi*(q2 - q3))/180)))/180, -(pi*cos((pi*q1)/180)*(l2*cos((pi*q2)/180) + l3*cos((pi*(q2 - q3))/180) + l4*cos((pi*(q2 - q3 + q4))/180)))/180, (pi*cos((pi*q1)/180)*(l3*cos((pi*(q2 - q3))/180) + l4*cos((pi*(q2 - q3 + q4))/180)))/180, -(l4*pi*(cos((pi*(q1 + q2 - q3 + q4))/180) + cos((pi*(q1 - q2 + q3 - q4))/180)))/360]
[-(pi*cos((pi*q1)/180)*(l4*sin((pi*(q2 - q3 + q4))/180) + l2*sin((pi*q2)/180) + l3*sin((pi*(q2 - q3))/180)))/180, -(pi*sin((pi*q1)/180)*(l2*cos((pi*q2)/180) + l3*cos((pi*(q2 - q3))/180) + l4*cos((pi*(q2 - q3 + q4))/180)))/180, (pi*sin((pi*q1)/180)*(l3*cos((pi*(q2 - q3))/180) + l4*cos((pi*(q2 - q3 + q4))/180)))/180, -(l4*pi*(sin((pi*(q1 - q2 + q3 - q4))/180) + sin((pi*(q1 + q2 - q3 + q4))/180)))/360]
[0, -(pi*(l4*sin((pi*(q2 - q3 + q4))/180) + l2*sin((pi*q2)/180) + l3*sin((pi*(q2 - q3))/180)))/180, (pi*(l4*sin((pi*(q2 - q3 + q4))/180) + l3*sin((pi*(q2 - q3))/180)))/180, -(l4*pi*sin((pi*(q2 - q3 + q4))/180))/180]
[0, (pi*sin((pi*q1)/180))/180, -(pi*sin((pi*q1)/180))/180, (pi*sin((pi*q1)/180))/180]
[0, -(pi*cos((pi*q1)/180))/180, (pi*cos((pi*q1)/180))/180, -(pi*cos((pi*q1)/180))/180]
[pi/180, 0, 0, 0]

## Symbolic Jacobian J (per-radian)

Row 1:
[ sin(q1)*(l4*sin(q2 - q3 + q4) + l2*sin(q2) + l3*sin(q2 - q3)),
  -cos(q1)*(l2*cos(q2) + l3*cos(q2 - q3) + l4*cos(q2 - q3 + q4)),
   cos(q1)*(l3*cos(q2 - q3) + l4*cos(q2 - q3 + q4)),
  - (l4/2)*( cos(q1 + q2 - q3 + q4) + cos(q1 - q2 + q3 - q4) ) ]

Row 2:
[ -cos(q1)*(l4*sin(q2 - q3 + q4) + l2*sin(q2) + l3*sin(q2 - q3)),
  -sin(q1)*(l2*cos(q2) + l3*cos(q2 - q3) + l4*cos(q2 - q3 + q4)),
   sin(q1)*(l3*cos(q2 - q3) + l4*cos(q2 - q3 + q4)),
  - (l4/2)*( sin(q1 - q2 + q3 - q4) + sin(q1 + q2 - q3 + q4) ) ]

Row 3:
[ 0,
  - ( l4*sin(q2 - q3 + q4) + l2*sin(q2) + l3*sin(q2 - q3) ),
    ( l4*sin(q2 - q3 + q4) + l3*sin(q2 - q3) ),
  - l4*sin(q2 - q3 + q4) ]

Row 4:
[ 0, sin(q1), -sin(q1), sin(q1) ]

Row 5:
[ 0, -cos(q1), cos(q1), -cos(q1) ]

Row 6:
[ 1, 0, 0, 0 ]

## Forward Kinematics (numeric)

x = -0.1570 m
y = -0.0605 m
z = 0.3727 m

Euler ZYX (deg):
roll  = -90.0000°
pitch = -26.7800°
yaw   = -158.9300°

## Jacobian from differentiation (numeric)

0.0011   -0.0046    0.0029   -0.0008
-0.0027   -0.0018    0.0011   -0.0003
0         -0.0029    0.0013   -0.0017
0          0.0063   -0.0063    0.0063
0         -0.0163    0.0163   -0.0163
0.0175     0         0         0

## Inverse Velocity Kinematics

Desired v = [0.019036  -0.006411  -0.013865] m/s

qdot (deg/s) = [-4.477675  2.306787  2.771328  4.811676]

Reconstructed v = [0.019036  -0.006411  -0.013865] m/s

-220.3858   44.0510    0.0000

-1.3446   -6.7270 -165.1104

-85.5201 -427.8540 -119.4602

-86.9873 -435.1946 -265.2387

# Appendices

Fig1. Desired velocities on simscape

Fig2. Forward Position

Fig3. Forward Velocity Differentiation

Fig4. output velocity on transform sensor

Fig5. Desired Positions on simscape

Fig6. Output Position on transform sensor

Fig7. Robotic arm

Fig8. Circuitry